PASO 1

Agregar un router mod 1941, 1 switch mod 2960, y 2 computadoras

PASO 2

Conectar el cable de red de la pc1 al puerto fa0/1 del switch

Conectar el cable de red de la pc2 al puerto fa0/2 del switch

Conectar el cable de red al puerto GigabitEthernet0/1 del switch al puerto GigabitEthernet0/0 del Router

PASO 3

Configurar las tarjetas de red de la pc0 IP: 172.17.10.10 255.255.255.0 Gateway: 172.17.10.1

Configurar las tarjetas de red de la pc1 IP: 172.17.20.10 255.255.255.0 Gateway: 172.17.20.1

PASO 4

Crear VLANS en el switch

S1>enable

S1#configure terminal

S1(config)# vlan 10

S1(config-vlan)# name estudiante

S1(config-vlan)# end

S1(config)# vlan 20

S1(config-vlan)# name docente

S1(config-vlan)# end

PASO 5

Asignar puertos a cada VLANS

S1#configure terminal

S1(config)# interface fa0/1

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 10

S1(config-if)# end

S1(config)# interface fa0/2

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 20

S1(config-if)# end

PASO 6

Revisar las vlan

S1#show vlan brief

PASO 7

Si ejecutan un ping entre las pc´s, va a mostrar error, ya que están en diferentes redes y diferentes vlans.

PASO 8

Configurar las subinterfaces en le Router con la encapsulación 802.1Q o dot1Q

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface Gigabitethernet0/0.1

Router(config-subif)#encapsulation dot1Q 10

Router(config-subif)#ip address 172.17.10.1 255.255.255.0

Router(config-subif)#interface Gigabitethernet0/0.2

Router(config-subif)#encapsulation dot1Q 20

Router(config-subif)#ip address 172.17.20.1 255.255.255.0

Router(config-subif)#end

Router#

PASO 9

Levantar o activar el puerto GigabitEhternet0/0 del router

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface Gigabitethernet0/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

%LINK-5-CHANGED: Interface GigabitEthernet0/0.1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0.1, changed state to up

%LINK-5-CHANGED: Interface GigabitEthernet0/0.2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0.2, changed state to up

Router(config-if)#end

Router#show ip interface brief

Router#

PASO 10

Habilitar el enlace troncal en el switch

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#interface gigabitethernet0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

Switch(config-if)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show interface trunk

Port Mode Encapsulation Status Native vlan

Gig0/1 on 802.1q trunking 1

Port Vlans allowed on trunk

Gig0/1 1-1005

Port Vlans allowed and active in management domain

Gig0/1 1,10,20

PASO 11

Probar conectividad entre las pc’s

PARA QUITAR PUERTOS DE UNA VLAN:

SW1# configure terminal
Enter configuration commands, one per line.  No command
SW1(config)# interface GigabitEthernet 0/1
SW1(config-if)# no switchport access vlan
SW1(config-if)# exit
SW1(config)# end

PARA QUITAR VLANS

switch# configure terminal
Enter configuration commands, ending with CTRL-Z or EXEC
switch(config)# no vlan 100
switch(config)# exit